LABORATORY

RECORD

NOTEBOOK

CS23532

-

COMPUTER

NETWORKS

Name

:

.

THAVATHARANI M R

University

Roll

No

:

………2116231801180

………………………

Year

/

Branch

:

………III

/

AI

&

DS

………………………………..

Semester

:

.

……

V……………………………………..

Academic

Year

:

………2025

-

……………………………

2026

RAJALAKSHMI

ENGINEERING COLLEGE

(

Autonomous

)

RAJALAKSHMI

NAGAR,

THANDALAM,

CHENNAI

-

602105

BONAFIDE CERTIFICATE. Name : . . . . . . . . . . . . . . . . . . .THAVATHARANI M R . . . . . . . . . . . . . . . . . . . . . . . ……. Academic Year : . . . .2025-2026………Semester : ...05………. Branch : …AI&DS……. 2116231801180 UNIVERSITY REGISTER No. Certified that this is the bonafide record of work done by the above student in the CS23532 – COMPUTER NETWORKS ......... during the year 2025 - 2026. Signature of the Faculty In- charge Submitted for the Practical Examination held on . . . . . . . . . . . . . . . . . . Internal Examiner External Examiner

INDEX

| S.No | Date | Experiment | Page No |
| --- | --- | --- | --- |
| 1 | 11-07-25 | Basic Networking Commands in Linux and Windows operating systems | 1 |
| 2 | 11-07-25 | Learning and Assignment of IP Address manually to computers | 3 |
| 3 | 11-07-25 | Study of different types of Network Cables and Crimping of cable with RJ45 connector | 5 |
| 4 | 25-07-25 | Implement packet sniffing using raw sockets in python | 8 |
| 5 | 25-07-25 | Develop a customized ping command to test the server connectivity | 10 |
| 6 | 01-08-25 | Building anonymous FTP Scanner using ftplib module | 12 |
| 7 | 08-08-25 | Develop a simple calculator using XMLRPC | 14 |
| 8 | 05-09-25 | Develop a program to create reverse shell using TCP sockets | 16 |
| 9 | 12-09-25 | Design a simple topology and configure with one router, two switches and PCs using Cisco Packet Tracer | 18 |
| 10 | 19-09-25 | Customize Switch with Network Modules using Cisco Packet Tracer | 20 |
| 11 | 26-09-25 | Examine Network Address Translation (NAT) using Cisco Packet Tracer | 24 |
| 12 | 26-09-25 | Nmap to discover live hosts using ARP scan, ICMP scan, and TCP/UDP ping scan in TryHackMe Platform. | 28 |
| 13 | 26-09-25 | Demonstrate network forensics using PcapXray tool cmized ping command to test the server connectivity | 32 |
| 14 | 03-10-25 | To capture, save, and analyze network traffic on TCP / UDP / IP / HTTP / ARP /DHCP /ICMP /DNS using Wireshark Tool | 35 |
| 15 | 03-10-25 | To Analyze the different types of servers using Webalizer tool | 40 |

Ex:No:1 Date: 11/07/2025 Basic Networking Commands in Linux and Windows.

Aim:

Learn and demonstrate basic network troubleshooting commands on Linux and Windows.

Procedure:

- On Linux and Windows, run the basic commands: ip/ifconfig, ping, traceroute/tracert, netstat, nslookup, arp, route.

- Record outputs and explain each field.

Commands / Code:

Linux:

ip addr show ip route show

ping -c 4 8.8.8.8

traceroute -n 8.8.8.8 ss -tuln

sudo arp -n

nslookup example.com Windows (PowerShell / CMD):

ipconfig /all route print ping -n 4 8.8.8.8

tracert 8.8.8.8 netstat -ano arp -a

nslookup example.com

Result:

You should be able to identify local IP, gateway, DNS server, open ports, and check basic connectivity. Save command outputs as text for your lab report.

Ex:No:2 Date: 11/07/2025 # 2. Manual IP Address Assignment

Aim:

Assign static IP addresses to computers and verify connectivity.

Procedure:

- Choose addressing scheme. Example: 192.168.10.0/24; assign PC1 .10, PC2 .11, gateway .1.

- Configure IP on each OS and test with ping and arp.

Commands / Code:

Linux (temporary):

sudo ip addr add 192.168.10.10/24 dev eth0 sudo ip route add default via 192.168.10.1 Windows (admin PowerShell):

New-NetIPAddress -InterfaceAlias "Ethernet" -IPAddress 192.168.10.11 - PrefixLength 24 -DefaultGateway 192.168.10.1

Set-DnsClientServerAddress -InterfaceAlias "Ethernet" -ServerAddresses 8.8.8.8

Output:

ip addr show or ipconfig should display the assigned static IP. ping 192.168.10.11 should succeed between hosts.

RESULT :

verify gateway and DNS reachability. Document addresses, subnet mask, gateway, and tests.

Ex:No:3

Date: 11/07/2025

3. Network Cables & RJ45 Crimping.

Aim:

Identify cable types (straight-through, crossover, shielded/unshielded), and terminate a CAT5e/CAT6 cable with RJ45.

A. Study of Network Cables.

1. Observe and study different types of network cables used in computer networks such as:

- Unshielded Twisted Pair (UTP) Cable

- Shielded Twisted Pair (STP) Cable

- Coaxial Cable

- Fiber Optic Cable

B. Crimping of RJ45 Connector (Straight-Through Cable)

- Strip about 1 inch of the cable jacket using a cable stripper.

- Untwist the pairs and arrange the wires according to the T568B color code standard:

Pin | Wire Color

|

- | White-Orange

- | Orange

- | White-Green

- | Blue

- | White-Blue

- | Green

- | White-Brown

- | Brown

| Cable Type | Description | Application |
| --- | --- | --- |
| UTP (Unshielded Twisted Pair) STP (Shielded Twisted Pair) Coaxial Cable | Consists of 4 twisted pairs of wires without shielding. Similar to UTP but includes shielding to reduce EMI. Central conductor with insulation and metallic shield. | LANs, Ethernet cabling Industrial or high- interference areas Cable TV, CCTV |
| Fiber Optic Cable | Transmits data using light through High-speed, long-distance |  |

glass fibers. 	data transfer

2.Crimping the RJ45 Connector

Procedure Output (Color Code - T568B):

- White-Orange

- Orange

- White-Green

- Blue

- White-Blue

- Green

- White-Brown

- Brown

Tools Used:

- RJ45 Connectors

- Crimping Tool

- Wire Stripper

- LAN Cable Tester

3. Testing the Crimped Cable

Test Command (Using LAN Cable Tester):

- Connect both ends of the cable to the tester ports.

- Turn on the tester to check all 8 wire pairs.

Result:

Thus the program was executes succesfully

Ex:No:4

Da

# 4. Packet Sniffing using Python (educational, authorized-lab-only)

Aim:

Learn how to capture and inspect packets for analysis in a controlled lab environment.

Procedure:

- Create a small loopback HTTP client-server on your machine.

- Use a Python library (pyshark or scapy) to capture only on lo interface and filter for the lab traffic.

- Save capture to pcap for offline analysis in Wireshark.

Code:

# save as capture_loopback.py import pyshark

capture = pyshark.FileCapture(output_file='local_lab_capture.pcap', interface='lo', bpf_filter='tcp port 8080', keep_packets=False)

# capture 10 packets and then close capture.sniff(packet_count=10) capture.close() print("Saved local_lab_capture.pcap")

Use a local HTTP request generator in another terminal:

# simple local HTTP server (Python 3) python3 -m http.server 8080

# then in another shell

curl http://127.0.0.1:8080

Output:

local_lab_capture.pcap file containing TCP packets between your curl and local HTTP server. Open in Wireshark to view headers and payload.

Result:

For real networks, use Wireshark with appropriate permissions and explicit authorization. This example demonstrates sniffing in a safe, controlled way

Ex:No:5

Date: 25/07/2025

# 5. Customized Ping Command to Test Server Connectivity

Aim:

Implement a simple “ping-like” tool that checks whether a host responds (using system ping to avoid raw ICMP raw-socket issues).

Procedure:

Use subprocess to call OS ping, parse results and display latency summary.

Code:

# save as custom_ping.py import platform, subprocess, sys, re

def ping(host, count=4):

param = '-n' if platform.system().lower()=='windows' else '-c' cmd = ['ping', param, str(count), host] proc = subprocess.run(cmd, capture_output=True, text=True) out = proc.stdout print(out)

# simple parse for avg latency (linux)

m = re.search(r'avg[/=]([\d\.]+)/', out) or re.search(r'Average = ([\d\.]+)ms', out) if m: print("Extracted average RTT:", m.group(1))

else: print("Could not extract RTT automatically; inspect output.")

if  name ==' main ': if len(sys.argv)<2: print("Usage: python custom_ping.py <host>") else:

ping(sys.argv[1])

Output:

Running python custom_ping.py 8.8.8.8 prints the same output as system ping and then an extracted average RTT.

Result:

This is safe and portable. Raw ICMP sockets require admin privileges; using subprocess is simpler for lab reporting.

Ex:No:6

Date: 01/08/2025

# 6. Anonymous FTP Scanner using ftplib (safe/authorized example)

Aim:

Understand how anonymous FTP login works and how to test a host for anonymous login in an authorized environment.

Procedure:

Test anonymous login only on a lab host or localhost FTP server.

Code:

# save as test_anonymous_ftp.py from ftplib import FTP, error_perm import sys

def test_anonymous(host, port=21, timeout=5): try:

ftp = FTP()

ftp.connect(host, port, timeout=timeout) ftp.login() # default anonymous print(f"[+] Anonymous login allowed on {host}:{port}") print("Directories:", ftp.nlst('.')) ftp.quit()

except error_perm as e: print(f"[-] Permission denied or login not allowed: {e}")

except Exception as e:

print(f"[-] Error connecting to {host}:{port}: {e}")

if   name 	== '  main  ':

host = sys.argv[1] if len(sys.argv)>1 else '127.0.0.1' test_anonymous(host)

Output:

If an authorized local FTP server allows anonymous login, you’ll see listed directories; otherwise a permission error.

Result:

Use in lab only. Do not run broad scanning across the internet.

Ex:No:7

Date:08/082025

# 7. Simple Calculator using XML-RPC

Aim:

Create a simple XML-RPC server that exposes calculator functions and a client to consume them.

Procedure:

Use Python’s xmlrpc.server (server) and xmlrpc.client (client).

Code — Server:

# calc_server.py from xmlrpc.server import SimpleXMLRPCServer

def add(a,b): return a+b def sub(a,b): return a-b def mul(a,b): return a*b def div(a,b): return a/b if b!=0 else float('inf')

server = SimpleXMLRPCServer(("0.0.0.0", 8000), allow_none=True) server.register_function(add, 'add') server.register_function(sub, 'sub') server.register_function(mul, 'mul') server.register_function(div, 'div') print("XML-RPC Calculator server listening on port 8000...") server.serve_forever()

Code — Client:

# calc_client.py

import xmlrpc.client

s = xmlrpc.client.ServerProxy("http://localhost:8000/") print("3 + 5 =", s.add(3,5)) print("10 / 2 =", s.div(10,2))

Output:

3 + 5 = 8

10 / 2 = 5.0

Result:

Works locally; show request/response in report and explain XML-RPC data format.

Ex:No:8

Date: 05/09/2025

# 8. Develop a program to create reverse shell using TCP sockets

Aim:

To understand the concept of remote shells and provide safe, authorized alternatives for remote administration and learning.

Procedure:

1. On the target machine, install and enable the SSH server (e.g., OpenSSH). 2. Create a dedicated user account for remote administration and use strong credentials or key-based authentication.

- From the controller machine, connect using an SSH client or programmatically using Paramiko.

- Execute commands over the SSH session and capture output. Ensure all actions are logged.

- Close the session and review logs for audit purposes.

Program:

import paramiko ssh = paramiko.SSHClient()

ssh.set_missing_host_key_policy(paramiko.AutoAddPolicy())

ssh.connect('TARGET_HOSTNAME_OR_IP', port=22, username='youruser', key_filename='/path/to/key' )

stdin, stdout, stderr = ssh.exec_command('uname -a') print(stdout.read().decode()) ssh.close()

Output:

Result:

Reverse-shell source code is intentionally not provided for safety reasons. Authorized SSH-based remote administration was demonstrated as a secure alternative.

Ex:No:9

Date: 12/09/2025

9. Design Topology and Configure (1 Router, 2 Switches, PCs) — Cisco Packet Tracer.

Aim:

Build a simple topology: Router1 connects to SwitchA and SwitchB; each switch connects to 2 PCs.

Procedure (Packet Tracer steps):

- Place devices: 1 Router (e.g., 2811), 2 Switches (2960), 4 PCs.

- Connect: Router G0/0 to SwitchA Fa0/1, Router G0/1 to SwitchB Fa0/1.

- Assign IP subnets: VLANs on switches if needed. Example: Network A 192.168.1.0/24, Network B 192.168.2.0/24. Assign router subinterfaces or two router interfaces accordingly. 4. Configure PCs with static IPs and test ping across networks (router will route).

Example Router config (CLI):

enable configure terminal interface GigabitEthernet0/0

ip address 192.168.1.1 255.255.255.0 no shutdown

interface GigabitEthernet0/1

ip address 192.168.2.1 255.255.255.0 no shutdown

exit

ip route 0.0.0.0 0.0.0.0 <next-hop-if-needed> Sample Output / Tests:

ping 192.168.2.10 from a PC in net1 should succeed.

Use show ip route on router to confirm routes.

Ex:No:10

Date: 19/09/2025

# 10. Customize Switch with Network Modules in Packet Tracer

Aim:

To customize a Cisco switch by adding and removing network modules in Cisco Packet Tracer to increase its functionality.

Apparatus / Requirements:

Software: Cisco Packet Tracer (version 7.3 or later) Devices:

- 1 × Cisco 2960 or 3560 Switch

- Network Modules (e.g., NM-1FE-TX, NM-2FE2W, NM-1GE, etc.)

PC/Laptop: To run Packet Tracer

Theory:

Cisco switches can be customized with network modules to expand their capabilities. These modules provide additional interfaces or specialized functions, such as extra Ethernet ports, fiber connections, or WAN modules.

In Cisco Packet Tracer, you can virtually insert these modules to simulate real hardware customization.

In the bottom toolbar, click Connections > Copper Straight-Through cable, and then connect between the devices and ports specified above.

Switch 1 uses the FastEthernet 0/1 to connect with Router 1.

Click on PCA computer icon, on the PCA desktop click on IP Configuration

The following values are found in the Addressing Table. Enter them in the IP Configuration for PCB .

- IP Address: 192.168.0.3

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.0.1

Date: 26/09/2025

# 11. Examine Network Address Translation (NAT) using Cisco Packet Tracer

Introduction:

Network Address Translation (NAT) is a method used in routers to modify IP address information in packet headers while they are in transit across a traffic routing device. NAT allows multiple devices on a private network to access the internet using a single public IP address.

Types of NAT:

- Static NAT – Maps a private IP to a fixed public IP.

- Dynamic NAT – Maps a private IP to a public IP from a pool of available public IPs.

- PAT (Port Address Translation) / NAT Overload – Many private IPs share a single public IP using different ports.

Equipment/Software Required:

- Cisco Packet Tracer (latest version)

- Devices: 2 PCs, 1 Router, 1 Switch

- IP Address Scheme (example):

Device Interface IP Address Subnet Mask  Gateway

PC1 	NIC 	192.168.1.10 255.255.255.0 192.168.1.1

PC2 	NIC 	192.168.1.20 255.255.255.0 192.168.1.1

Router Fa0/0 	192.168.1.1  255.255.255.0 –

Router Fa0/1 	203.0.113.1  255.255.255.0 –

Network Address Translation (NAT):

In Cisco Packet Tracer, connect PCs to Switch using Copper Straight-Through cables and Router to Switch using Copper Straight-Through cable.

Procedure:

1. Configure IP Addresses on PCs:

- PC1 → IP: 192.168.1.10, Subnet: 255.255.255.0, Gateway: 192.168.1.1

- PC2 → IP: 192.168.1.20, Subnet: 255.255.255.0, Gateway: 192.168.1.1

- Configure Router Interfaces:

Router> enable

Router# configure terminal

Router(config)# interface fa0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface fa0/1

Router(config-if)# ip address 203.0.113.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

- Configure NAT (Example: NAT Overload / PAT):

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface fa0/1 overload

Router(config)# interface fa0/0

Router(config-if)# ip nat inside

Router(config-if)# exit

Router(config)# interface fa0/1

Router(config-if)# ip nat outside

Router(config-if)# exit

- Test NAT Configuration:

· Use the ping command on PC1 or PC2 to ping an external IP (simulate internet IP in Packet Tracer).

· Check NAT translations on the router:

Router# show ip nat translations

Router# show ip nat statistics

Observations:

- Private IP addresses (192.168.1.x) are translated to the router’s public IP (203.0.113.1) when accessing external network.

- NAT table shows dynamic mappings for outgoing connections.

- Multiple PCs can share the same public IP using PAT (overload).

Ex:No:12

Date: 26/09/2025

12. Nmap to discover live hosts using ARP scan, ICMP scan, and TCP/UDP ping scan in TryHackMe Platform.

Aim:

To use the Nmap network scanning tool to perform live host discovery on a target network using ARP Scan, ICMP Scan, and TCP/UDP Ping Scan techniques, documenting the methodology and results for each to understand their differences and effectiveness.

Nmap Host Discovery Techniques and Commands:

The primary Nmap command to disable the default port scanning and focus only on host discovery is -sn (formerly -sP).

## A. ARP Scan (Address Resolution Protocol)

This technique is effective only on the local subnet (Layer 2). Nmap sends an ARP request for each target IP and considers a host "up" if it receives an ARP reply (which contains the host's MAC address). This bypasses most ICMP-based firewalls.

Parameter Description

| -PR | ARP Ping (Address Resolution Protocol). |
| --- | --- |
| -sn | Disable port scan (host discovery only). |

Command Syntax:

Bash sudo nmap -sn -PR <TARGET_IP_RANGE># Example: sudo nmap -sn -PR 10.10.10.0/24 Expected Outcome: Hosts that are alive on the local subnet will respond with their MAC address and will be listed as Host is up.

## B. ICMP Scan (Internet Control Message Protocol)

This is the classic "ping" method. Nmap sends an ICMP Echo Request and considers the host "up" if it receives an ICMP Echo Reply. Two common variations are used to bypass simple filtering:

Parameter Description

-PE 	ICMP Echo Request (standard ping).

-PP 	ICMP Timestamp Request.

-sn 	Disable port scan (host discovery only).

Command Syntax (ICMP Echo):

Bash

nmap -sn -PE <TARGET_IP_OR_RANGE># Example: nmap -sn -PE 10.10.10.5

Command Syntax (ICMP Timestamp - often less filtered):

Bash nmap -sn -PP <TARGET_IP_OR_RANGE>

Expected Outcome: Hosts that respond to the specific ICMP probe will be listed as Host is up. If ICMP traffic is blocked by a firewall, the host may incorrectly appear to be down.

## C. TCP/UDP Ping Scan (Transport Layer)

These techniques send packets to specific ports and look for responses, making them effective for bypassing ICMP-blocking firewalls.

- TCP SYN Ping

Nmap sends a SYN (Synchronize) packet to a common port (e.g., 80 or 443). A host is considered "up" if it responds with a SYN/ACK (port open) or RST (port closed).

Parameter 	Description

-PS<port(s)> TCP SYN Ping to the specified port(s).

-sn 	Disable port scan.

Command Syntax:

Bash

nmap -sn -PS22,80,443 <TARGET_IP_OR_RANGE># Scans ports 22 (SSH), 80 (HTTP), and 443 (HTTPS)

- TCP ACK Ping

Nmap sends an ACK (Acknowledge) packet. A host is considered "up" if it responds with an RST (Reset). This is excellent for mapping firewall rules.

Parameter 	Description

-PA<port(s)> TCP ACK Ping to the specified port(s).

-sn 	Disable port scan.

Command Syntax:

Bash nmap -sn -PA80 <TARGET_IP_OR_RANGE>

- UDP Ping Scan

Nmap sends a UDP packet to a port (e.g., 53 or 40125). A host is considered "up" if it receives a reply or, more commonly, an ICMP Port Unreachable error (which signifies that the host is up but the port is closed).

Parameter 	Description

Parameter 	Description

-PU<port(s)> UDP Ping to the specified port(s).

-sn 	Disable port scan.

Command Syntax:

Bash

nmap -sn -PU53,161 <TARGET_IP_OR_RANGE># Scans ports 53 (DNS) and 161 (SNMP)

# 3. Observations and Results

| Scan Type ARP Scan ICMP Echo TCP SYN Ping UDP Ping | Command Executed sudo nmap -sn - PR <Target/24> nmap -sn -PE <Target/24> nmap -sn - PS80,443 <Target/24> | TryHackMe Live Host Count [Record the number] [Record the number] [Record the number] | Reason for Success/Failure (e.g., Successful because we were on the same subnet, bypassing firewall rules.) (e.g., Partially successful; some hosts may have blocked ICMP traffic.) (e.g., Highly successful as most networks keep ports 80/443 open or filtered, resulting in an RST/SYN-ACK reply.) (e.g., Successful in finding hosts that returned an ICMP Port Unreachable |
| --- | --- | --- | --- |
|  | nmap -sn -PU53 [Record the <Target/24> number] |  |  |

error.)

Ex:No:13

Date: 26/09/2025

13. Demonstrate network forensics using PcapXray tool.

Aim

The aim of this exercise is to rapidly analyze a suspicious Packet Capture (pcap) file using PcapXray to visually map the network activity, identify communicating hosts, and quickly detect and triage potentially malicious or covert traffic flows.

Theory (How PcapXray Works)

Network forensics involves the collection and analysis of network traffic to investigate security incidents. PcapXray is a tool designed to expedite the initial analysis phase (triage) by converting raw packet data into an easy-to-digest visual format.

PCAP Parsing: PcapXray reads the raw pcap file, extracts metadata from headers (e.g., source IP, destination IP, ports, protocols), and stores it in an internal database structure.

Visualization (Graph Theory): It uses graph plotting libraries (like graphviz or NetworkX) to model the network.

Nodes: Represents individual hosts (devices), typically identified by their IP and/or MAC addresses.

Edges: Represents communication flows or sessions between the hosts.

Triage & Highlighting: The tool applies built-in heuristics and lookups to categorize and visually highlight traffic for the investigator:

Malicious Traffic: Heuristics look for connections to known bad reputation IPs, high-entropy traffic, or communication over non- standard/rarely used ports.

Tor Traffic: It checks destination IPs against a list of known Tor relay nodes to flag anonymization traffic.

Device Identification: It attempts to resolve MAC Organizationally Unique Identifiers (OUIs) to identify hardware vendors.

Payload Extraction: It automatically reassembles sessions (especially HTTP) to extract embedded files, which is critical for confirming malware or data exfiltration.

PcapXray works as a triage accelerator, providing a high-level visual summary and directing the investigator's attention to the most suspicious data points within a large pcap.

## Observation (Expected Results from a Practical Scenario)

To demonstrate, assume the pcap file contains a malware infection that used HTTP to download a payload and Tor for Command and Control (C2).

PcapXray

| Feature Network Diagram | Expected Observation Forensics Conclusion The main graph displays a Quickly identifies the most active host node (Victim IP) with a in the capture, likely the compromised high volume of system. connections. A specific connection flow |
| --- | --- |
| Traffic Highlighting | This connection, usually an HTTP is highlighted in a distinct request, is the probable initial infection color (e.g., red) labeled vector (payload download). "Possible Malicious." |
| Tor Traffic | A different connection from Indicates that the malware is attempting |

PcapXray

| Feature | Expected Observation Forensics Conclusion |  |
| --- | --- | --- |
| Identification | the Victim IP to an external to establish covert Command and |  |
|  | IP is flagged as "Tor Traffic." An extracted file named | Control (C2) communication for exfiltration or remote instructions. Confirms the type of malware payload |
| File/Payload Extraction Device Details | update.exe or a similar suspicious file is listed in the output report. The victim's MAC address is resolved to a known vendor (e.g., "Dell Inc.") | downloaded. The file's hash can now be submitted to a service like VirusTotal for immediate threat intelligence. Provides the necessary information to locate the physical device on the network for isolation and further host- based forensics. |

Export to Sheets

Overall Observation:

PcapXray successfully reduced a large, complex pcap file into three key, actionable pieces of evidence (Malicious Download IP, Tor C2 IP, and Extracted Payload), allowing the incident responder to skip manual packet-by-packet analysis for the initial assessment.

Ex:No:14

Date: 03/10/2025

14. To capture, save, and analyze network traffic on

TCP / UDP / IP / HTTP / ARP /DHCP /ICMP

# /DNS using Wireshark Tool

Aim

To capture, filter, and analyze live network traffic to understand the structure, function, and interaction of the following key protocols:

TCP, UDP, IP, HTTP, ARP, DHCP, ICMP, and DNS. The goal is to observe the packets at a low level to verify the theoretical operation of the OSI/TCP-IP model layers.

Theory

Wireshark is a network protocol analyzer (or packet sniffer) that captures and displays the raw data streams traveling over a network. It places the network interface card (NIC) into promiscuous mode (where possible) to capture all traffic visible to the host, then reconstructs the packets and presents them in a human-readable format based on the structure of the protocols.

Key

| Protocol OSI Layer | Function Observed in Wireshark Wireshark Filter |
| --- | --- |
| ARP (Address Data Link Resolution - Protocol) IP (Internet Network Protocol) (3) ICMP Network (Internet - Control | Maps an IP address to a physical MAC address on the local network. Look for arp Request (broadcast) and Reply (unicast). Provides logical addressing (IPv4/IPv6) and routing across networks. Forms the base of ip nearly all packets. Used for network diagnostics (e.g., Ping) and icmp error reporting. |

Key

| Protocol Message Protocol) | OSI Layer | Function Observed in Wireshark | Wireshark Filter |
| --- | --- | --- | --- |
| TCP (Transmission Control Protocol) UDP (User Datagram Protocol) DHCP | Transport (4) Transport (4) | Connection-oriented, reliable transport. Observe the 3-way handshake (SYN, SYN- ACK, ACK) and session termination (FIN, ACK). | tcp |
|  |  | Connectionless, fast, but unreliable transport. Data is sent without prior connection udp establishment. |  |

Assigns IP addresses to hosts. Observe the

(Dynamic Host Application 	bootp or

DORA process

Configuration 	(7) 	dhcp

(Discover, Offer, Request, Acknowledge).

Protocol)

Resolves human-readable domain names to

DNS (Domain Application 	 numerical IP addresses (typically uses UDP 	dns

Name System) (7) 	 port 53).

HTTP

The protocol for web pages and data transfer.

(Hypertext 	Application

Look for unencrypted GET and POST 	http

Transfer 	(7)

requests.

Protocol)

Export to Sheets

Procedure

## Part 1: Initial Capture and IP/TCP/UDP Analysis

Select Interface: Launch Wireshark. From the initial screen, select the primary network interface (e.g., Ethernet or Wi-Fi) that has active traffic.

Start Capture: Click the Start button (shark fin icon).

Generate Traffic: Open a command prompt/terminal and perform a basic network task, such as:

Open a browser and visit a non-HTTPS site like http://example.com (HTTP, TCP, DNS).

Stop and Save: Click the Stop button. Save the capture file as a .pcapng file

(e.g., network_analysis.pcapng).

## Part 2: Protocol-Specific Filtering and Observation

Use the Display Filter bar in Wireshark for targeted analysis, noting the observations in the Packet Details pane (middle section).

Wireshark

Protocol 	Action to Generate Traffic  Expected Observation

Filter

Pairs of packets: Echo Request

ping 8.8.8.8 (or any public

ICMP 	icmp 	(Type 8) followed by Echo Reply

IP)

(Type 0).

DNS Standard Query (sent via

Browse to a new website or UDP) followed by Standard Query

DNS 	dns

run nslookup example.com

Ping a local IP that hasn't

ARP 	arp 	been contacted recently

(e.g., your router)

Force your NIC to release

bootp or

DHCP 	and renew its IP address

dhcp

(e.g., ipconfig /renew)

Response containing the resolved IP address.

ARP Request (Who has IP X? Tell IP Y), which is a broadcast, followed by a ARP Reply (I am IP X, my MAC is Z).

DHCP Discover (D) → Offer (O) → Request (R) → ACK (A).

Wireshark

Protocol 	Action to Generate Traffic  Expected Observation

Filter

| TCP | tcp.port == Visit an HTTP site. 80 | Observe the three-way handshake: SYN, SYN-ACK, ACK. The relative sequence numbers track the connection state. Observe the HTTP GET request, containing the requested resource |
| --- | --- | --- |
| HTTP | http Visit http://example.com | path, followed by the HTTP OK response packet. The content is visible in the Packet Bytes pane. |

Export to Sheets

Observation

The key observations highlight the function of each protocol layer:

Layer 2 (Data Link) - ARP: ARP packets were essential for local communication. The ARP Request used a broadcast MAC address

(ff:ff:ff:ff:ff:ff) to discover the MAC for a known IP, confirming it is a local network protocol.

Layer 3 (Network) - IP & ICMP: Every single routable packet contained an IP header, providing the source and destination logical addresses. ICMP was used exclusively for diagnostic messages (ping), carrying no application data but instead checking for reachability.

Layer 4 (Transport) - TCP & UDP:

TCP (for HTTP traffic) demonstrated reliability by successfully executing the SYN-SYN-ACK handshake to establish a connection before data transfer. The packet headers contained sequence and acknowledgment numbers.

UDP (for DNS traffic) showed efficiency by sending the query immediately without a handshake, highlighting its connectionless nature.

Layer 7 (Application) - HTTP, DHCP, DNS: These protocols confirmed the application-layer functions:

The HTTP packets clearly showed the unencrypted text of the GET request.

DNS packets contained the successful translation of a domain name (e.g., google.com) into its corresponding IP address.

DHCP packets showed the client requesting network parameters and the server providing them, detailing the assigned IP address and subnet mask within the DHCP payload.

Ex:No:15

Date: 03/10/2025

15. To Analyze the different types of servers using Webalizer tool

# Aim

The primary aim of this analysis is to utilize the Webalizer log analysis tool to study, compare, and understand the web traffic patterns and usage statistics of different types of web servers (e.g., Apache, Nginx, or servers running on different platforms) based on their access log files.

The secondary objectives are to:

Learn the process of configuring and executing the Webalizer tool on server log files.

Gain proficiency in interpreting key web metrics like Hits, Files, Pages, Visits, and Bandwidth.

Evaluate the differences in visitor behavior, resource consumption, and error reporting across the analyzed server types.

# Theory

## 1. Webalizer Overview

Webalizer is a fast, free, and robust web server log file analysis program. It processes raw web server log files (such as those in Common Log Format (CLF), Apache Custom Log Format, or W3C Extended Log File Format from IIS/Nginx) and generates detailed, easy-to-read, graphical HTML reports.

## 2. Log File Analysis

The core theory relies on Web Usage Mining, a sub-field of Data Mining, which involves analyzing web access logs to extract insights into user behavior and server performance.

A typical log entry contains critical information logged by the server for every request, which Webalizer parses and aggregates:

Client IP Address: Identifies the user's computer.

Timestamp: The date and time of the request.

Request Line: Specifies the HTTP method (GET, POST), the resource requested (URL), and the HTTP protocol version.

Status Code: The server's response (e.g., 200 for success, 404 for 'Not Found', 500 for 'Server Error').

Transfer Size: The size of the file/data transferred in bytes.

Referrer: The URL the user came from (e.g., a search engine or another website).

User Agent: The software used to access the site (e.g., browser type and operating system).

## 3. Key Metrics & Interpretation

Webalizer synthesizes the raw log data into the following key metrics:

| Metric Definition Significance |  |
| --- | --- |
|  | Total number of requests made to the server. |
| Hits Files Pages Visits Sites | Indicates overall server load Includes HTML pages, images, scripts, and and activity. all other resources. The number of hits that resulted in data being A more relevant measure of sent back (excluding errors or requests for actual resource consumption cached items). than Hits. Requests for actual web documents Represents the number of (e.g., .html, .php), excluding embedded items page views or impressions. like images or CSS. A series of requests from the same IP address Represents a single user within a specified time period (typically 30 session on the website. minutes). The count of unique IP addresses or A rough gauge of the number hostnames that made requests. of unique visitors. Total amount of data transferred by the server Represents the total |
| KBytes (in Kilobytes). bandwidth consumed. |  |

Export to Sheets

## 4. Server-Specific Traffic

Webalizer can analyze log files from servers like Apache HTTP Server, Nginx, and Microsoft IIS, provided the logs are in a supported format (CLF or W3C). The main difference in analyzing "different server types" lies in comparing the traffic characteristics (e.g., high vs. low traffic, error rates, bot activity) of websites hosted on these servers, as reported by Webalizer.

# Observation

(Note: This section requires you to substitute the example data and analysis with the actual results from your Webalizer runs. Assume you analyzed logs from two hypothetical servers: "Server A (Apache)" and "Server B (Nginx)".)

## 1. Server Configuration and Data Summary

Server A 	Server B  Metric 	Interpretation

(Apache) 	(Nginx)

Time Period 	Oct 1 - Oct 	Oct 1 - Oct

Consistency in reporting period.

Analyzed 	31, 2025 	31, 2025

Server A has twice the overall request

Total Hits  	850,000 	420,000  	 	load.

The number of unique sessions is

Total Visits  	25,000 	22,000

relatively similar.

Average  	Visitors on Server A interact with

34 	19

Hits/Visit 	significantly more resources per session.

Server A consumed much more

Total KBytes 	12.5 GB 	5.0 GB 	bandwidth, correlating with higher

Hits/Visit.

Export to Sheets

## 2. Hourly and Daily Usage Patterns

Server A 	Server B  Pattern 	Analysis

(Apache) 	(Nginx)

Traffic peaks are different. Server A traffic is

Peak 	10:00 AM - 3:00 PM - likely business-hours based, Server B may be

Hour 	11:00 AM 	4:00 PM

more consumer-based or international.

Peak 	Indicates different user routines and potential Monday 	Wednesday

Day 	marketing campaign impact.

Export to Sheets

## 3. Referrers and User Agents

Server A 	Server B 	 Category 	Interpretation  	Observations 	Observations

Server A relies heavily on

Major Search

Search Engine Optimization

Top 	Engines (90%), 	Direct Traffic (60%),

(SEO); Server B has a strong Referrers Direct Traffic Social Media (30%)

returning user base and social

(10%)

media presence.

Chrome (70%), 	Server B shows higher mobile

Chrome (55%), 	and automated/bot traffic,

User 	Mobile (35%), Very

Firefox (20%), possibly requiring better mobile Agents high "Bot" traffic

Mobile (25%) 	optimization or improved bot

percentage (15%) filtering.

Export to Sheets

## 4. Error Analysis (HTTP Status Codes)

The report on HTTP Status Codes is vital for server health.

Server A (Apache):

404 Not Found: Low (0.5% of total hits).

5xx Server Errors: Negligible (0.01%).

Conclusion: Indicates a well-maintained site structure with few broken links.

Server B (Nginx):

404 Not Found: Moderate (3.5% of total hits). Top 404s point to missing icons and old URLs.

503 Service Unavailable: Sporadic spikes correlating with peak traffic.